Project Report

- Phase 1 – Problem Framing & Layout Assets

  - Milestone01/ only contains binary deliverables (CAD models, a literature-review DOCX, and the stage video), which indicates the project started with workspace research, UR5e spec study, and a detailed CAD mock-up of the sorting workcell before any code was written. Because these files are binary, there are no inline line numbers to cite, but they anchor the initial “define + model” step.

- Phase 2 – Foundational Kinematics & ROS 2 Hook-Up

  - A Denavit–Hartenberg forward-kinematics solver for UR5e is introduced in Milestone02/codes/fk_ur5e.py (lines 1-32), and the same parameter set feeds a numerical IK routine in Milestone02/codes/ik_ur5e.py (lines 1-75). These scripts establish a consistent geometric model that later milestones reuse.

  - Milestone02/codes/ik_controller.py (lines 1-61) wraps MuJoCo in a ROS 2 node that publishes joint states at 100 Hz, proving the simulator↔ROS data bridge works and setting the stage for closed-loop control.

  - Analysis: this step nails the math/ROS plumbing but still lacks velocity or acceleration reasoning, so joint-speed limits and smooth trajectories are not enforced yet.

- Phase 3 – Full Velocity/Acceleration Kinematics + Validation

  - Milestone03/README.md (lines 1-215) documents the deliverables: the upgraded codes/kinematics_derivations.py implements FK, Jacobian, J̇, and forward/inverse velocity/acceleration mappings; codes/test_sim_vs_kinematics.py validates them; and a MuJoCo GUI plus scene XML provide visualization.

  - The actual implementation derives Jacobians and their derivatives numerically, and maps between twists and joint states (Milestone03/codes/kinematics_derivations.py (lines 1-145)).

  - The repository-level test suite (tests/README.md (lines 3-76) and tests/test_sim_vs_kinematics.py (lines 1-140)) compares analytic results against MuJoCo for 5 joint configurations with millimeter-level tolerances, giving quantitative evidence that the math is correct.

  - Analysis: This milestone closes the modeling loop by proving IK/velocity/acceleration math against the simulator, but it still assumes zero-noise measurements and scripted targets.

- Phase 4 – Trajectory Planning & Task-Space Controller

  - The cubic trajectory generator in Milestone04/trajectory_planner.py (lines 1-150) (duplicated later in src/mujoco_ros2/mujoco_ros2/trajectory_planner.py (lines 1-151)) enforces zero-velocity boundary conditions, producing joint or Cartesian profiles with continuous velocity and acceleration.

  - A ROS 2 “TrajectoryController” node (Milestone04/ik_controller.py (lines 1-156) and the installed copy src/mujoco_ros2/mujoco_ros2/ik_controller.py (lines 1-156)) fuses FK for the current pose, the task-space trajectory, a proportional position loop, and inverse-velocity kinematics to stream smooth joint-velocity commands.

  - Analysis: This step upgrades the system from point-to-point IK jumps to executable motion plans with tunable gains; however, it still assumes a constant tool orientation and does not yet include collision constraints or timing synchronization with process equipment.

- Phase 5 – Industrial Scenario & PID Layer

  - src/mujoco_ros2/mujoco_ros2/industrial_application.py (lines 1-166) repackages the planner into a higher-level welding state machine (move-to-start → weld → retract), adds per-joint PID wrappers, and demonstrates how to sequence multiple Cartesian segments from ROS 2.

  - Analysis: The PID layer mostly clamps commanded velocities (the feedforward term dominates), so further tuning would be needed for torque-level control or for handling external disturbances.

- Phase 6 – Sorting Station Automation

  - The project README dedicated to the sorting cell (SORTING_STATION_README.md (lines 3-150)) captures the environment layout, running modes, user controls, IK/trajectory-based automation loop, and future enhancements such as vision and conveyors.

  - The executable simulation (src/mujoco_ros2/mujoco_ros2/sorting_station_gui.py (lines 1-200)) resolves the scene XML, preplans a “sky pose”, sequences four pick-and-place jobs with individualized hover/pick/place points, runs cubic joint trajectories at 150 Hz, and manages virtual grasping to lock boxes to the gripper frame.

  - launch_sorting_station.py (lines 1-22) provides a zero-setup entry point for demonstrations.

  - Analysis: This is the Milestone 5 deliverable—an end-to-end automation script that closes the loop from planning through execution. It is deterministic (scripted pickup/drop poses, no perception), so adding sensing or variability would be the next frontier. The future-work checklist in SORTING_STATION_README.md (lines 115-123) already flags conveyors, multi-arm coordination, and statistics dashboards.

- Phase 7 – Regression & Packaging

  - Tests live both under Milestone03/codes/test_sim_vs_kinematics.py and the repo-level tests/test_sim_vs_kinematics.py, making it easy to rerun the quantitative checks after any model or controller change (tests/README.md (lines 18-58)).

  - The ROS 2 package in src/mujoco_ros2/mujoco_ros2/ consolidates all reusable modules (FK, IK, planners, controllers, industrial logic), so colcon builds can install them for other launch files.

  - Analysis: Maintaining the same math stack in both milestone folders and the installable package keeps behavior consistent, but it also means changes must be synchronized manually across copies to avoid drift.

Overall Analysis

- Mathematical fidelity is strong: FK/IK/velocity/acceleration pipelines are validated numerically and reused everywhere, reducing the risk of silent divergence between the simulator and controllers.

- Planning/control hierarchy is modular (trajectory planner → task-space controller → PD joint loop in the GUI), making it straightforward to retarget to other tasks, but orientation planning is still hard-coded to a single “tool-down” frame.

- The final sorting demo is deterministic and lacks sensing, which is acceptable for Milestone 5 but leaves open work for robustness (vision, error recovery) as highlighted in the README’s future-work list.

- Tests cover kinematics but not the full manipulation routine; adding scenario-level regression (e.g., verifying all boxes end in the correct bins) would improve confidence when modifying the GUI script.

Next Steps

- Prioritize the README’s future items: start with conveyor animation/logic and a simple vision mock to introduce variability.

- Extend the regression suite to cover the automated sorting choreography (e.g., pose tolerances for each drop) before making behavioral changes.